Fall

Amplitude Modulation

Project

2022

Communication

Name               	 		Sec 			BN

Zeyad Tarek           		1 			28

Abdelrahman Hamdy	1			36

Explanation of our work:

Starting off by reading our 3 input signals from the “Input” folder, we make sure that the sampling frequency in all these signals is the same for consistent modulation. Then, we choose the first channel for our messages and find the length of each one along with the maximum length. An extra step is done which I found helped in eliminating noise and greatly reducing the overlap between the 3 signals which is to up-sample the signals with a sample ratio of 3 before modulation and then down sample after demodulation to restore the signal back to its original state. This is done using a function called “resample” which takes 3 arguments: The signal itself, x, and y. The sample rate is basically multiplied by the ratio of x/y. In order to be able to sum all modulated signals of each input, we need to make sure that they all have the same length so the next crucial step is to set their lengths to this maximum length and fill the gaps with zeros. We also adjust the time and frequency intervals on this max length and the original sampling frequency.

Carrier Frequency:

The carrier frequency is set to be the width of the signal with the largest width to ensure that when the signals are shifted during modulation, they do not overlap and fail to restore the input message signals. As for the second carrier frequency, it has to satisfy a condition where it covers the other signals by 2 * Bandwidth plus another half of the width which is the same carrier frequency of the first. As a conclusion, we have two frequencies to work with: Fc_1 which is the half width of the message signal drawn in the frequency domain and Fc_2 which is triple that width.

Modulation:

We perform Double-Sideband Suppressed Carrier (DSB-SC) modulation on the first input signal with a cosine carrier of frequency Fc_1. As for the second and third input signals, we use QAM (Quadrature amplitude modulation) where two signals are sent simultaneously over the same bandwidth (on the same carrier frequency Fc_2) for efficient use of the bandwidth, but one is a sine function and the other is cosine. The difference in phase between these 2 functions helps us restore them separately without having to worry about the overlap that could happen.

What about the final modulated signal?

Now this should be the easiest step as all we have to do is sum the three modulated signals obtained from the DSB-SC and QAM to get our resulting modulated signal, ready for being used for demodulation.

Demodulation:

Our passband frequency for the low pass filter is the same as the original carrier frequency as all we need to do is filter each signal alone and output it for testing. For the synchronous demodulation with zero phase shift and frequency addition, the LPF takes the final modulated signal, the sampling frequency and the passband frequency. The signal is multiplied by 3 carriers, one for each input signal, obtaining an output of half the original signal in amplitude so we simply finish by multiplying by 2 and passing the necessary parameters mentioned above to the low pass filter to get our audio back again.

Demodulation with phase shift:

The same exact steps done with the synchronous demodulation will be done here but the only difference is that we add a phase shift with the carrier inside the cosine function. As required, we have a phase shift of 10, 30, and 90 degrees (all converted to radians by a simple calculation of course) which will affect the output signal. Phase shift causes attenuation of the signal which means decreasing its amplitude. However, there is a special case for phase shift 90 because it causes the first signal to be almost gone and can’t be heard as cosine (90) is zero. For the other 2 signals, we used QAM by applying cosine and sine for the same frequency and when adding a 90 degree phase shift, the two signals interchange. One takes place of the other and vice versa because 90 phase shift causes cosine to become sine and the opposite as well.

Demodulation with a different local carrier frequency:

A constant is added with the carrier frequency that is calculated with Omega and then we observe the effect of that on the carrier when it’s multiplied by the modulated signal. For this, we have 2 cases where we add this constant on Fc: 2Hz and 10 Hz. The effect adding a 2Hz constant is distortion of the signal and a bit of noise will be clearly heard in the background but we would still be able to identify the audio from the main core of the signal as the effect wasn’t that much drastic. On the other hand, with 10 Hz the signal will be mostly disrupted by random noises and we would fail to know which audio is currently playing.

Message 1 Signal:

Message 2 Signal:

Message 3 Signal:

Message 1 Modulated Signal:

Message 2 Modulated Signal:

Message 3 Modulated Signal:

Sum of the three modulated input signals:

Message 1 Demodulated signal with synchronous carrier:

Message 2 Demodulated signal with synchronous carrier:

Message 3 Demodulated signal with synchronous carrier:

Message 1 demodulated PhaseShift10:

Message 2 demodulated PhaseShift10:

Message 3 demodulated PhaseShift10:

Message 1 demodulated PhaseShift30:

Message 2 demodulated PhaseShift30:

Message 3 demodulated PhaseShift30:

Message 1 demodulated PhaseShift90:

Message 2 demodulated PhaseShift90:

Message 3 demodulated PhaseShift90:

Message 1 demodulated Carrier frequency increased by 2 Hz:

Message 2 demodulated Carrier frequency increased by 2 Hz:

Message 3 demodulated Carrier frequency increased by 2 Hz:

Message 1 demodulated Carrier frequency increased by 10 Hz:

Message 2 demodulated Carrier frequency increased by 10 Hz:

Message 3 demodulated Carrier frequency increased by 10 Hz:

Code:

% Close all opened figures

close all;

% Read the input audio

[message_1, fs_1] = audioread('Input/audio_1.m4a');

[message_2, fs_2] = audioread('Input/audio_2.m4a');

[message_3, fs_3] = audioread('Input/audio_3.m4a');

% Set sample ratio

global RESAMPLE_FACTOR;

RESAMPLE_FACTOR = 3;

% Select first channel for the three signals & Up-sample

message_1 = resample(message_1(:, 1), RESAMPLE_FACTOR, 1);

message_2 = resample(message_2(:, 1), RESAMPLE_FACTOR, 1);

message_3 = resample(message_3(:, 1), RESAMPLE_FACTOR, 1);

% Set Sampling Frequency (They are all the same)

fs = fs_1;

% Get length of the 3 signals

N_1 = length(message_1);

N_2 = length(message_2);

N_3 = length(message_3);

% Get the maximum length among them

max_N = max([N_1, N_2, N_3]);

% Adjust all lengths of the 3 input signals to be the same (having max length)

message_1 = [message_1;zeros(max_N-N_1, 1)];

message_2 = [message_2;zeros(max_N-N_2, 1)];

message_3 = [message_3;zeros(max_N-N_3, 1)];

% Time & Frequency intervals

time = linspace(0, max_N/fs, max_N);

df = fs/2;

freq = -df : fs/max_N: df - fs/max_N;

% Amplitude and phase in frequency domain

message_f1 = fftshift(fft(message_1));

message_f2 = fftshift(fft(message_2));

message_f3 = fftshift(fft(message_3));

phase_1 = unwrap(angle(message_f1));

phase_2 = unwrap(angle(message_f2));

phase_3 = unwrap(angle(message_f3));

% Carrier Frequency

fc_1 = 5000;

fc_2 = 3 * fc_1;

% Omega = 2piF

wc_1 = 2*pi * fc_1;

wc_2 = 2*pi * fc_2;

% Carrier Equation in time domain

carrier_t1 = cos(wc_1 * time);

carrier_t2 = cos(wc_2 * time);

carrier_t3 = sin(wc_2 * time);

% Carrier amplitude and phase in frequency domain

carrier_f1 = fftshift(fft(carrier_t1));

carrier_f2 = fftshift(fft(carrier_t2));

carrier_f3 = fftshift(fft(carrier_t3));

phase_carrier1 = unwrap(angle(carrier_f1));

phase_carrier2 = unwrap(angle(carrier_f2));

phase_carrier3 = unwrap(angle(carrier_f3));

% Apply DSB-SC Modulation

modulatedSignal_t1 = message_1' .* carrier_t1;

modulatedSignal_t2 = message_2' .* carrier_t2;

modulatedSignal_t3 = message_3' .* carrier_t3;

% Modulated Signals in frequency domain

modulatedSignal_f1 = fftshift(fft(modulatedSignal_t1));

modulatedSignal_f2 = fftshift(fft(modulatedSignal_t2));

modulatedSignal_f3 = fftshift(fft(modulatedSignal_t3));

phase_mod1 = unwrap(angle(modulatedSignal_f1));

phase_mod2 = unwrap(angle(modulatedSignal_f2));

phase_mod3 = unwrap(angle(modulatedSignal_f3));

% Add modulated signals

finalModulatedSignal = modulatedSignal_t1 + modulatedSignal_t2 + modulatedSignal_t3;

% Get modulated signal amplitude and phase in frequency domain

finalModulatedSignal_f = fftshift(fft(finalModulatedSignal));

phase_mod = unwrap(angle(finalModulatedSignal_f));

% Plot Message Signals

draw(time, freq.', 1, message_1, abs(message_f1), phase_1, 'Message 1');

draw(time, freq.', 2, message_2, abs(message_f2), phase_2, 'Message 2');

draw(time, freq.', 3, message_3, abs(message_f3), phase_3, 'Message 3');

% Plot Modulated Signals

draw(time, freq.', 4, modulatedSignal_t1, abs(modulatedSignal_f1), phase_mod1, 'Modulated 1');

draw(time, freq.', 5, modulatedSignal_t2, abs(modulatedSignal_f2), phase_mod2, 'Modulated 2');

draw(time, freq.', 6, modulatedSignal_t3, abs(modulatedSignal_f3), phase_mod3, 'Modulated 3');

% Plot the Final Modulated Signal

draw(time, freq.', 7, finalModulatedSignal, abs(finalModulatedSignal_f), phase_mod, 'FINAL Modulated Signal');

% Figure counter for Plotting

global j;

j = 8;

% Set passband frequency

fp = fc_1;

% Synchronous Demodulation

demodulate(finalModulatedSignal, carrier_t1, fs, fp, "Sync_1");

demodulate(finalModulatedSignal, carrier_t2, fs, fp, "Sync_2");

demodulate(finalModulatedSignal, carrier_t3, fs, fp, "Sync_3");

% Phase Shift = 10

[carrier1_10, carrier2_10, carrier3_10] = generateCarriersWithPhase(fc_1, fc_2, time, 10);

demodulate(finalModulatedSignal, carrier1_10, fs, fp, "PhaseShift10_1");

demodulate(finalModulatedSignal, carrier2_10, fs, fp, "PhaseShift10_2");

demodulate(finalModulatedSignal, carrier3_10, fs, fp, "PhaseShift10_3");

% Phase Shift = 30

[carrier1_30, carrier2_30, carrier3_30] = generateCarriersWithPhase(fc_1, fc_2, time, 30);

demodulate(finalModulatedSignal, carrier1_30, fs, fp, "PhaseShift30_1");

demodulate(finalModulatedSignal, carrier2_30, fs, fp, "PhaseShift30_2");

demodulate(finalModulatedSignal, carrier3_30, fs, fp, "PhaseShift30_3");

% Phase Shift = 90

[carrier1_90, carrier2_90, carrier3_90] = generateCarriersWithPhase(fc_1, fc_2, time, 90);

demodulate(finalModulatedSignal, carrier1_90, fs, fp, "PhaseShift90_1");

demodulate(finalModulatedSignal, carrier2_90, fs, fp, "PhaseShift90_2");

demodulate(finalModulatedSignal, carrier3_90, fs, fp, "PhaseShift90_3");

% Demodulation with local carrier frequency different than Fc by 2 Hz

[carrier1_2Hz, carrier2_2Hz, carrier3_2Hz] = generateCarriersWithDifferentFc(fc_1, fc_2, time, 2);

demodulate(finalModulatedSignal, carrier1_2Hz, fs, fp, "Fc2Hz_1");

demodulate(finalModulatedSignal, carrier2_2Hz, fs, fp, "Fc2Hz_2");

demodulate(finalModulatedSignal, carrier3_2Hz, fs, fp, "Fc2Hz_3");

% Demodulation with local carrier frequency different than Fc by 10 Hz

[carrier1_10Hz, carrier2_10Hz, carrier3_10Hz] = generateCarriersWithDifferentFc(fc_1, fc_2, time, 10);

demodulate(finalModulatedSignal, carrier1_10Hz, fs, fp, "Fc10Hz_1");

demodulate(finalModulatedSignal, carrier2_10Hz, fs, fp, "Fc10Hz_2");

demodulate(finalModulatedSignal, carrier3_10Hz, fs, fp, "Fc10Hz_3");

function draw(time, freq, i, mt, mf, phase, T)

% Plot Message in Time Domain

figure(i)

subplot(3, 1, 1)

plot(time, mt)

xlabel('Time')

ylabel('Amp')

title(strcat(T, ' Signal (Time)'))

% Plot Message Amplitude in Frequency Domain

subplot(3, 1, 2)

plot(freq, mf)

xlabel('Freq')

ylabel('Amp')

title(strcat(T, ' Signal Amplitude (Freq)'))

% Plot Message Phase in Frequency Domain

subplot(3, 1, 3)

plot(freq, phase)

xlabel('Freq')

ylabel('Phase')

title(strcat(T, ' Signal Phase (Freq)'))

end

function demodulate(signal, carrier, fs, fp, filename)

% Re-multiply by the carrier to restore the signal with half magnitude

% So we double the output to get the exact original signal

demodulatedSignal = 2 * (signal .* carrier);

% Low pass filter to get each message alone

lpf = lowpass(demodulatedSignal, fp, fs);

% Down-sample the signal back to its original Fs

global RESAMPLE_FACTOR;

lpf = resample(lpf, 1, RESAMPLE_FACTOR);

% Get demodulated signal amplitude and phase in Frequency Domain

lpf_f = fftshift(fft(lpf));

phase_demod = unwrap(angle(lpf_f));

% Plot the demodulated signal

N = length(lpf);

global j;

time = linspace(0, N/fs, N);

freq = -(fs/2) : fs/N: (fs/2) - fs/N;

draw(time, freq.', j, lpf, abs(lpf_f), phase_demod, filename);

j = j + 1;

% Output the final result to an audio file

audiowrite(strcat("Output/", filename, '.m4a'), lpf, fs);

end

function [carrier1, carrier2, carrier3] = generateCarriersWithPhase(fc_1, fc_2, time, deg)

% Phase Shift in radians

phaseShift = (deg * pi) / 180;

% Omega = 2piF

wc_1 = 2*pi * fc_1;

wc_2 = 2*pi * fc_2;

% Get carrier for each signal with an additional phase shift

carrier1 = cos((wc_1 * time) + phaseShift);

carrier2 = cos((wc_2 * time) + phaseShift);

carrier3 = sin((wc_2 * time) + phaseShift);

end

function [carrier1, carrier2, carrier3] = generateCarriersWithDifferentFc(fc_1, fc_2, time, dfc)

% Get new omega with a constant added to the carrier frequency

wc_1 = 2*pi * (fc_1 + dfc);

wc_2 = 2*pi * (fc_2 + dfc);

% Get new carriers just like we did in synchronous demodulation

carrier1 = cos(wc_1 * time);

carrier2 = cos(wc_2 * time);

carrier3 = sin(wc_2 * time);

end